ME4233 Part 2 CA 2

AY 2024/2025

A0266108E Justin Leonard

A0306098Y Wang Zeqi

Abstract

In this assignment, the main objective is to solve the lid-driven cavity problem by using explicit and implicit method. These two methods are used to check the difference in stability for explicit and implicit method.

Finite Difference Method

In order to solve the lid-driven cavity problem, the Navier-Stokes equation in the streamfunction- vorticity formulation is used.

∂ξ∂t+∂ψ∂y ∂ξ∂x-∂ψ∂x∂ξ∂y= 1Re+∂2ξ∂x2+∂2ξ∂y2

∇2ψ= -ξ

For the spatial discretization, the central finite difference is used for both equations whereas the time advancement, the explicit Euler method and implicit Euler method are used. In this study, the Re is set to be 28. Additionally, the grid resolution is Nx × Ny = 50 × 20.

First of all, the Poisson equation is discretized as shown below

Δx=1Nx=150           Δy=1Ny=120

Г2=(Δx)2(Δy)2

∂2ξ∂x2+∂2ξ∂y2= -ξ

ψi+1,j  n-2* ψi,j  n+ ψi-1,j nΔx2+ψi,j+1  n-2* ψi,j  n+ ψi,j-1 nΔy2= -ξ

Rearranging the equation will give

1(Δx)2 ψi+1,j  n+ 1(Δx)2 ψi-1,j  n+Г2(Δx)2 ψi,j+1   n+ Г2(Δx)2 ψi,j-1   n- 2*Г2+1Δx2 ψi,j-1   n= -ξ

The left-hand side term will then be used to construct the A matrix to solve the Poisson equation, while the vorticity will be the right-hand side matrix.

Au = b

Where u is the new streamfunction. This Poisson equation will then be solved using the SOR method with omega (ɷ) = 1.5. From there we will get the new streamfunction to calculate the vorticity at the next timestep.

Explicit Euler Method

For the time advancement, the initial time difference that is used dt = 0.001 and the final time tf = 2. Therefore, the maximum timestep can be calculated as tf dt=2000.

Based on the stability condition,

Δt . 1Re1Δx2+ 1Δx2< 12

Δt . 1Re1Δx2+ 1Δx2< 12

Δt < 0.020656

Hence, the maximum delta t for the stable system is Δt < 0.020656. After applying the explicit Euler method and the central finite difference for the Navier-Stoke equation, it will become

ξi,j  n+1- ξi,j  nΔt= RHSn

Where the RHSn,

-ψi,j+1n- ψi,j-1  n 2 Δyξi+1,j  n- ξi-1,j  n2 Δx+ψi+1,1n- ψi-1,j  n 2 Δxξi,j+1  n- ξi,j-1  n2 Δy+ 1Re ξi+1,j  n-2* ξi,j  n+ ξi-1,j nΔx2+ξi,j+1  n-2* ξi,j  n+ ξi,j-1 nΔy2

Moreover, the boundary conditions should also be calculated to determine the value of both streamfunction and vorticity at the boundary. Since u = v = 0 at the wall, and v = 0 and u = sin(2пft) at the open boundary, where f = 5. There, the boundary conditions can be calculated as follows

South Boundary

Normal velocity v= - ∂ψ∂x=0 , streamfunction ψ at south boundary = 0

Tangential velocity u= ∂ψ∂yS=0

Vorticity ξS= - ∂2ψ∂y2S

North Boundary

Normal velocity v= - ∂ψ∂x=0 , streamfunction ψ at north boundary = 0

Tangential velocity u= ∂ψ∂yS= sin(2пft)

Vorticity ξS= - ∂2ψ∂y2N

West Boundary

Normal velocity u= ∂ψ∂y=0 , streamfunction ψ at west boundary = 0

Tangential velocity u= -∂ψ∂xW= 0

Vorticity ξS= - ∂2ψ∂y2W

East Boundary

Normal velocity u= ∂ψ∂y=0 , streamfunction ψ at east boundary = 0

Tangential velocity u= -∂ψ∂xE= 0

Vorticity ξS= - ∂2ψ∂y2S

After putting all the necessary parameters and boundary conditions, the energy array can be seen as below.

Figure 1. The energy density and vector field for explicit method (dt = 0.001)

Figure 2. Streamfunction and vorticity plot at the final timestep (dt = 0.001)

Unfortunately, when we put Δt to be bigger than 0.020656, the instability starts to occur. It can be observed from the energy array below where in the early timestep, there are significant fluctuations. Additionally, the iteration also stopped before reaching the maximum time step. This shows that the system becomes unstable when the Δt is set to be bigger than Δt value set by the stability criterion.

Figure 4. The unstable energy density and vector field for explicit method (dt = 0.03)

Implicit Euler Method

For the implicit Euler Method, there is a slight difference on the Navier-Stoke equation where it becomes,

ξi,j  n+1- ξi,j  nΔt= RHSn+1

Where the RHSn+1,

-ψi,j+1n- ψi,j-1  n 2 Δyξi+1,j  n+1- ξi-1,j  n+12 Δx+ψi+1,1n- ψi-1,j  n 2 Δxξi,j+1  n+1- ξi,j-1  n+12 Δy+ 1Re ξi+1,j  n+1-2* ξi,j  n+1+ ξi-1,j n+1Δx2+ξi,j+1  n+1-2* ξi,j  n+1+ ξi,j-1 n+1Δy2

Rearranging the right-hand side will give

1Δx2 -Г(ψi,j+1n- ψi,j-1  n) (ξi+1,j  n+1- ξi-1,j  n+1) 4+Г(ψi+1,1n- ψi-1,j  n) (ξi,j+1  n+1- ξi,j-1  n+1) 4+ 1Re ξi+1,j  n+1-2* ξi,j  n+1+ ξi-1,j n+1 +Г2 (ξi,j+1  n+1-2* ξi,j  n+1+ ξi,j-1 n+1)

Then multiply the entire equation with 4 will give

4Cξi,j  n+1-4C ξi,j  n= RHSn+1

Where the RHSn+1

-(Гψi,j+1n- Гψi,j-1  n-4Re )  ξi+1,j  n+1+(Гψi,j+1n- Гψi,j-1n+4Re) ξi-1,j n+1+(Гψi+1,1n- Гψi-1,1n+ 4Г2Re  ) ξi,j+1  n+1-(Гψi+1,1n- Г(ψi-1,1n-4Г2Re) ξi,j-1 n+1-  8 (Г2+1)Reξi,j  n+1

Subsequently, vorticity with timestep n+1 will be moved to the LHS and vorticity with timestep n will be moved to the RHS.

4C+ 8Re  1+ Г2ξi,j  n+1 + (Гψi,j+1n- Гψi,j-1  n-4Re )  ξi+1,j  n+1- (Гψi,j+1n- Гψi,j-1n+4Re) ξi-1,j  n+1 - (Гψi+1,1n- Гψi-1,1n+ 4Г2Re  ) ξi,j+1  n+1+ (Гψi+1,1n- Г(ψi-1,1n-4Г2Re) ξi,j-1 n+1= 4C ξi,j  n

This requires us to create another Au = b equation, where the A matrix will be the coefficient of the left-hand side vorticity, the u vector is the vorticity at timestep n+1, and the b vector is the vorticity at timestep n. When using the implicit method for dt = 0.03, which is bigger that the dt given by the stability criterion, the energy array is shown below.

Figure 5. The energy array and vector field for implicit method (dt = 0.03)

Additionally, the iteration also manages to reach the maximum timestep, which is 66 for dt = 0.03 (2/0.03).

Figure 6. Streamfunction and vorticity plot at the final timestep (dt = 0.03)

Task c FFT

The FFT is often used in numerical simulations to analyse the spectrum of a signal to understand its frequency characteristics. For physical phenomena with periodic or oscillatory characteristics, the FFT can help to study their main frequency components and harmonic structures. In fluid mechanics and mechanical engineering, FFT can be used to analyse signals such as velocity, pressure, etc. that vary with time, and extract frequency information for further physical analysis or model calibration.

The output of the FFT contains amplitude and phase information for each frequency:

- Amplitude: indicates the strength of this frequency component. In a spectrogram, the dominant frequency usually corresponds to the largest amplitude, while other smaller peaks may correspond to harmonics or noise.

- Frequency: The frequency with the largest amplitude corresponds to the dominant frequency of the signal, which reflects the periodicity of the signal.

Numerical Implementation

Time series analysis at point C (1.5,0.5)

Figure 7. The frequency plot for explicit method (dt = 0.001)

Figure 8. The frequency plot for implicit method (dt = 0.03)

- Left Plot (u-velocity)

- Steady periodic oscillation after t≈1 between 0.03 and -0.03, means a recurring flow fluctuation around Point C.

- Both plots show that there are 10 waves in the span of 2 seconds. Hence, the frequency at point C is indeed 5.

- Right Plot (FFT)

- The curve shows the significant frequency component is concentrated in the lower frequency range.

- There is noticeable peak near 5Hz on this plot, which show the dominant frequency.

Conclusion

- Explicit method is easier to be implemented. However, it is limited by the stability criterion where the value of Δt cannot be set to be huge. Alternatively, implicit method can be used to achieve a stability while using a higher Δt

- This periodic fluctuations at low frequencies indicate the changes of northern boundary conditions.

- The right plot shows a clearer spectrum with no high frequencies interference components indicating the velocity fluctuations at this point are stable and no significant noise interference about this system.

This analysis confirms that our numerical simulation accurately captures the periodic flow behaviour and reveals the dominant frequency matches the driving force.